Chapter 7B: Bluetooth Mesh Protocol

Time 1 Hour

This chapter covers the details of the Bluetooth Mesh Stack and Packets.

7B.1	MESH Stack Architecture	2

7B.1.1 Bearer Layer	2

7B.1.2 Network Layer	2

7B.1.3 Lower Transport Layer	3

7B.1.4 Upper Transport Layer	3

7B.1.5 Access Layer	3

7B.1.6 Foundation Model Layer	3

7B.1.7 Model Layer	3

7B.2	(Advanced) Packet Details	4

7B.2.1 Access Messages	4

7B.2.2 Control Messages	6

7B.2.3 Packet Segmentation and Reassembly	8

7B.3	Exercises	9

Exercise 7B.1 Mesh Profile Spec	9

Exercise 7B.2 Mesh Model Spec	9

# MESH Stack Architecture

Like all complex systems, Bluetooth mesh uses a layered approach. In this case, the stack is broken into 7 layers as shown below. The lowest layer sits on top of the BLE stack.

## Bearer Layer

The lowest level defines how mesh messages get to/from the BLE stack. Presently, there are two bearer layers – the Advertising Bearer and the GATT Bearer. Most devices in a mesh network will use the Advertising Bearer to send messages using BLE advertising packets.

The GATT Bearer allows devices that don't support the Advertising Bearer (such as most smartphones) to communicate indirectly with the network via GATT Proxy nodes using the Proxy Protocol.

## Network Layer

The network layer defines message address types and network message format. It can support multiple bearers, each of which may have multiple network interfaces including the local interface which is used for communication between elements in the same node. Then network layer determines which network interface(s) to output messages over.

An input filter is used to determine if messages from the bearer layer should be delivered to the network layer for processing or not. An output filter is used to determine if messages should be delivered to the bearer layer or should be dropped.

The Relay and Proxy features are implemented in the network layer. Therefore, any messages not intended for this node are relayed to the appropriate network interface(s) but are not sent to the next layer in the stack.

## Lower Transport Layer

The lower transport layer is responsible for segmenting outgoing messages so that they will fit in the required transport PDU (Protocol Data Unit) and reassembling segmented incoming messages.

## Upper Transport Layer

The upper transport layer is responsible for encryption, decryption, and authentication of application data being passed to/from the access layer. It is also responsible for generating and dealing with transport control messages such as friendship and heartbeat messages.

## Access Layer

The access layer defines how higher-level applications can make use of the upper transport layer. It includes defining the format of the application data, defining and controlling the encryption and decryption process performed in the upper transport layer, and verifying data from the upper transport layer is intended for the right network and application before sending it further up the stack.

## Foundation Model Layer

The foundation model layer implements models required to configure and manage a mesh network.

## Model Layer

The model layer implements the behaviors, messages, states, etc. as defined in the mesh model specification. The user application interacts with devices by using the models.

# (Advanced) Packet Details

Bluetooth mesh networks are accomplished using Advertising packets. The Advertising packet is 31 octets, but much of that space is used up by network overhead. That overhead includes information from the network layer, information from the lower transport layer, and information from the upper transport layer.

As was discussed in the prior chapter, the number of octets available for a message payload depends on whether it is a control or access messages and on whether the message is segmented or unsegmented.

The packets include message integrity checks for the network layer (for all messages) called the NetMIC and at the upper level transport layer (only for access messages) called the TransMIC. These are used to verify that a message was not modified (either intentionally or unintentionally) during transmission. The two fields for integrity checks are as follows:

| Name | Message Type | Size (Octets) |
| --- | --- | --- |
| NetMIC | Control | 8 |
| NetMIC | Access | 4 |
| TransMIC | Unsegmented Access | 4 |
| TransMIC | Segmented Access | 4 or 8 |

## Access Messages

### Unsegmented Access Messages

Unsegmented access messages can contain up to 11 octets of payload. The full 31 octets in the advertising packet (assuming a max size packet) are allocated as shown in the table below including the level in the stack that is responsible for each item.

| Octet | # of Octets | Field Name | Level | Notes |
| --- | --- | --- | --- | --- |
| 0 | 1 | Length | BLE ADV Packet | Advertising Packet Length |
| 1 | 1 | Type | BLE ADV Packet | Advertising Packet Type = Mesh |
| 2 | 1 | IVI ¦ NID | Network | 1 bit: LSB of IV Index 7 bits: value derived from NetKey to identify the encryption and privacy keys used for the packet |
| 3 | 1 | CTL ¦ TTL | Network | 1 bit: indicate an access message (0) 7 bits: Time to Live (TTL) |
| 4 – 6 | 3 | SEQ | Network | Sequence Number |
| 7 – 8 | 2 | SRC | Network | Source Address |
| 9 - 10 | 2 | DST | Network | Destination Address |
| 11 | 1 | SEG ¦ AKF ¦ AID | Lower Level Transport | 1 bit: unsegmented message (0) 1 bit: application key flag 6 bits: application key identifier |
| 12 – 22 | 11 | Payload | Payload | Message Payload including 1, 2, or 3 bytes of Opcode |
| 23 - 26 | 4 | TransMic | Upper Level Transport |  |
| 27 - 30 | 4 | NetMIC | Network | Message Integrity Check for Network layer (4 octets for Access messages) |

### Segmented Access Messages

Segmented access messages can contain 12 octets of payload for each packet except the last one which can contain up to either 4 or 8. The maximum payload for the last packet depends on the size of the Transport Message Integrity Check (TransMIC) which can be either 4 or 8 octets depending on the message. This allows a maximum of 380 or 376 octets of payload in a single access message depending on the last packet. One, two, or three of those octets is an opcode for the message.

| Number of Packets | Max Payload Size (Octets) |  |
| --- | --- | --- |
|  | 4 octet TransMIC | 8 octet TransMIC |
| 1 (unsegmented) | 11 | n/a |
| 1 (segmented) | 8 | 4 |
| 2 | 20 | 16 |
| n | (nx12)-4 | (nx12)-8 |
| 32 | 380 | 376 |

The full 31 octets in each advertising packet (assuming all max size packets) are allocated as shown in the table below including the level in the stack that is responsible for each item.

| Octet | # of Octets | Field Name | Level | Notes |
| --- | --- | --- | --- | --- |
| 0 | 1 | Length | BLE ADV Packet | Advertising Packet Length |
| 1 | 1 | Type | BLE ADV Packet | Advertising Packet Type = Mesh |
| 2 | 1 | IVI ¦ NID | Network | 1 bit: LSB of IV Index 7 bits: Value derived from NetKey to identify the encryption and privacy keys used for the packet |
| 3 | 1 | CTL ¦ TTL | Network | 1 bit: Indicates an access message (0) 7 bits: Time to Live (TTL) |
| 4 – 6 | 3 | SEQ | Network | Sequence Number |
| 7 – 8 | 2 | SRC | Network | Source Address |
| 9 - 10 | 2 | DST | Network | Destination Address |
| 11 | 1 | SEG ¦ AKF ¦ AID | Lower Level Transport | 1 bit: segmented message (1) 1 bit: application key flag 6 bits: application key identifier |
| 12 – 14 | 3 | SZMIC ¦ SeqZero ¦ SegO ¦ SegN | Lower Level Transport | 1 bit: Size of TransMIC 13 bits: Least significant bits of SeqAuth 5 bits: Segment offset number 5 bits: Last segment number |
| 15 – 18 15 – 22 15 – 26 | 4 8 12 | Payload | Payload | Message Payload including 1, 2, or 3 bytes of Opcode. The last packet is a max of either 4 or 8 octets depending on the size of TransMic. The other packets are 12 octets each since TransMic is sent only at the end of the last packet. |
| 19 – 26 23 – 26 | 8 4 | TransMic | Upper Level Transport | Either 4 or 8 octets depending on value of SZMIC. This is only included in the last packet. |
| 27 – 30 | 8 | NetMIC | Network | Message Integrity Check for Network layer |

## Control Messages

### Unsegmented Control Messages

Like unsegmented access messages, unsegmented control messages can contain up to 11 octets of payload, but the overall contents of the packet are slightly different. The full 31 octets in the advertising packet (assuming a max size packet) are allocated as shown in the table below including the level in the stack that is responsible for each item.

| Octet | # of Octets | Field Name | Level | Notes |
| --- | --- | --- | --- | --- |
| 0 | 1 | Length | BLE ADV Packet | Advertising Packet Length |
| 1 | 1 | Type | BLE ADV Packet | Advertising Packet Type = Mesh |
| 2 | 1 | IVI ¦ NID | Network | 1 bit: LSB of IV Index 7 bits: Value derived from NetKey to identify the encryption and privacy keys used for the packet |
| 3 | 1 | CTL ¦ TTL | Network | 1 bit: Indicates a control message (1) 7 bits: Time to Live (TTL) |
| 4 – 6 | 3 | SEQ | Network | Sequence Number |
| 7 – 8 | 2 | SRC | Network | Source Address |
| 9 – 10 | 2 | DST | Network | Destination Address |
| 11 | 1 | SEG ¦ Opcode | Lower Level Transport | 1 bit: Unsegmented message (0) 7 bits: Opcode |
| 12 – 22 | 11 | Payload | Payload | Message Payload |
| 23 – 30 | 8 | NetMIC | Network | Message Integrity Check for Network layer |

### Segmented Control Messages

Segmented control messages can contain up to 8 octets of payload each. Therefore, a control message can be at most 256 octets long.

| Number of Packets | Max Payload Size (Octets) |
| --- | --- |
| 1 (unsegmented) | 11 |
| 1 (segmented) | 8 |
| 2 | 16 |
| n | n*8 |
| 32 | 256 |

The full 31 octets in each of the advertising packets (assuming all max size packets) are allocated as shown in the table below including the level in the stack that is responsible for each item.

| Octet | # of Octets | Field Name | Level | Notes |
| --- | --- | --- | --- | --- |
| 0 | 1 | Length | BLE ADV Packet | Advertising Packet Length |
| 1 | 1 | Type | BLE ADV Packet | Advertising Packet Type = Mesh |
| 2 | 1 | IVI ¦ NID | Network | 1 bit: LSB of IV Index 7 bits: Value derived from NetKey to identify the encryption and privacy keys used for the packet |
| 3 | 1 | CTL ¦ TTL | Network | 1 bit: Indicates a control message (1) 7 bits: Time to Live (TTL) |
| 4 – 6 | 3 | SEQ | Network | Sequence Number |
| 7 – 8 | 2 | SRC | Network | Source Address |
| 9 – 10 | 2 | DST | Network | Destination Address |
| 11 | 1 | SEG ¦ Opcode | Lower Level Transport | 1 bit: Segmented message (1) 7 bits: Opcode |
| 12 – 14 | 3 | RFU ¦ SeqZero ¦ SegO ¦ SegN | Lower Level Transport | 1 bit: Unused 13 bits: Least significant bits of SeqAuth 5 bits: Segment offset number 5 bits: Last segment number |
| 15 – 22 | 8 | Payload | Payload | Message Payload |
| 23 – 30 | 8 | NetMIC | Network | Message Integrity Check for Network layer |

## Packet Segmentation and Reassembly

As mentioned previously, the segmentation and reassembly of packets are handled by the lower transport layer. An example of how segmentation is done can be seen in the figure below.

(This figure is taken from the Bluetooth Mesh Profile Specification)

# Exercises

Mesh Profile Spec

Download and review the Mech Profile Spec. Answer the following questions:

- What is the endianness for the network layer, lower transport layer, upper transport layer, and access layer?

Answer:

Network is big endian

Lower Transport is big endian

Upper Transport is big endian

Access is little endian

- What is a mesh gateway?

Answer: A mesh gateway is a node that is able to translate messages between the mesh network and a non-Bluetooth technology. A node may be able to send and receive mesh messages through a mesh gateway while not in range of any of the Relay nodes.

For example, a smartphone may use a cell network or a WiFi network to communicate with a mesh network via a gateway.

- For a Mesh Proxy Service, what characteristics are required and what properties must they have?

Answer:

Characteristic: Mesh Proxy Data In 	Properties: Write Without Response

Characteristic:	Mesh Proxy Data Out	Properties: Notify

Mesh Model Spec

Download and review the Mesh Model Spec. Answer the following questions:

- What is the difference between the states "Light Lightness Actual" and "Light Lightness Linear"? How are they related?

Answer: Light Lightness Linear is just a linear value of the brightness of a light. For example, it may represent the duty-cycle for a PWM driving an LED. Light Lightness Linear is adjusted so that it represents perception of brightness. This mapping is done because the human eye does not perceive brightness linearly. Therefore, to make a light appear a certain percentage of its maximum brightness a translation must be done. Light Lightness Linear allows the user to specify an intended brightness for human perception which is then translated to Light Lightness Actual to drive a PWM.

The relationship is:  Light Lightness Actual=65535√Light Lightness Linear65535

- How many lighting server models are defined?

Answer: There are 11 Lighting Server models. They can be found in sections 6.4.1 – 6.4.11 of the Mesh Model specification.

- List one set of messages that can be used for setting the Hue, Saturation (i.e. color) and Lightness of a light. Is this the only way to do it?

Answer: There are many ways to do this depending on the client and server implemented. For example:

- Light HSL Set

- Light HSL Hue Set, Light HSL Saturation Set, and Light Lightness Linear Set

- Generic Level Set (use individually on each element assuming different colors are different elements)

- Generic Delta Set (use individually on each element assuming different colors are different elements)

- Generic Move Set (use individually on each element assuming different colors are different elements)