Chapter 7A: Bluetooth Mesh Introduction

# Mesh Network Fundamentals

## Overview

- BLE is point to point – GAP Central and GAP Peripheral

- In a mesh, all devices can communicate with all other devices (direct or indirect).

- Relays make longer distances possible.

- Messages are sent using advertising packets.

- Devices in a mesh network are called "nodes".

- Devices that are not part of a mesh network (yet) are called "unprovisioned devices".

# Mesh Specs

- Show the mesh specs very briefly

# Nodes

- Discuss picture and each type of node – relay, GATT proxy, friend, low power

- Message caching, TTL

# Provisioning and Configuration/Management

To get a new device up and running on a Bluetooth Mesh network, it must be:

- Provisioned

- Configured

## Provisioning

Provisioning is the process by which a device is made a member of the mesh network and becomes a node. Needs:

- The network key (and other associated network security information like the IV index)

- A unicast address assigned to its primary element.

Discuss picture:

- Scan

- Beacon

- Provision Process:

  - Invitation and response

  - Exchanging Public Keys

  - Authentication

  - Distribution of Provisioning Data

    - Network key, IV index, Unicast address

## Configuration/Management

Distribution of application keys, assigning group addresses to models, etc.

# Client Applications

Concentrate on Android app

# Demo

3 dimmable LEDs, 1 switch

Switch and 2 dimmable LEDs provisioned, 1 LED not provisioned

Add 3rd light

Add room

Move 1 light to another room

Assign switch to room