Chapter 7C: Bluetooth Mesh in WICED

# Firmware Architecture

## Core Mesh Functionality

The main BT Mesh functionality libraries/mesh_app_lib. The user does not need modify anything.

## User Application

- Setup/config

- Callbacks – library handles advertising, BT callback, etc.

### Configuration

- Global variables

- Models (one per element) (1 array entry per model)

- Sensors (one per element) (1 array entry per sensor)

- Properties (one per element) (1 array entry per property)

- Elements (points to models, sensors, properties) (1 array entry per element)

- Device Config (points to elements)

### User Function Callbacks

wiced_bt_mesh_app_func_table_t wiced_bt_mesh_app_func_table =

{

mesh_app_init,          // application initialization

NULL,                   // Default SDK platform button processing

NULL,                   // GATT connection status

mesh_app_attention,     // attention processing

NULL,                   // notify period set

NULL,                   // WICED HCI command

NULL,                   // LPN sleep

NULL                    // factory reset

};

#### Application Initialization

- Called once when stack is running

- Passed a parameter for is_provisioned

- Init the models, sensors, properties and register callbacks

#### Button processing

- Default is a factory reset (remove provisioning info and config). User can override this behavior

#### GATT connection status

- Callback for GATT connection up/down

#### Attention processing

- Index of element and time is provided. App decides what to do (blink LED, sound a buzzer…)

#### Notify Period Set

- Called to indicate to the application that it needs to periodically send updates

- Typically used for sensors to configure the cadence

#### WICED HCI command

- This function is used if the application needs to interpret HCI commands received from a host device.

#### Low Power Node (LPN) Sleep

- Stack says HID_OFF is OK for the specified time

- Application determines wither or not to go into HID_OFF and when to wake up.

#### Factory Reset

- Default is to remove provisioning and config info

- If user gives a function, this is called BEFORE the reset. Can be used to erase app NVRAM data.

### Sending Messages

- The callbacks are used for receiving messages

- To send messages, use the appropriate function

### Notifying the Mesh Model Library of Local Changes

- If local hardware can change a state, need to notify the Mesh Model library on the device

- This is not required for sensor models because in that case the stack will call a report callback function whenever it receives a get message from a client

# MESH Demo Starter Applications

- Demos (built-in) and Snips (online)

- Show in MTB IDE

## BLE_MESH_LightDimmable (Server)

### Configuration

| Models | WICED_BT_MESH_DEVICE WICED_BT_MESH_MODEL_USER_PROPERTY_SERVER WICED_BT_MESH_MODEL_LIGHT_LIGHTNESS_SERVER |
| --- | --- |
| Sensors | N/A |
| Properties | WICED_BT_MESH_PROPERTY_DEVICE_FIRMWARE_REVISION |
| Features | WICED_BT_MESH_CORE_FEATURE_BIT_FRIEND WICED_BT_MESH_CORE_FEATURE_BIT_RELAY WICED_BT_MESH_CORE_FEATURE_BIT_GATT_PROXY_SERVER |

### User Application Functionality

light_dimmable.c and led_control.c/h.

callbacks for application initialization (mesh_app_init) and attention processing (mesh_app_attention)

mesh_app_init: The application initialization function does the following:

- Set the firmware version

- Initialize a 1 second timer for attention processing

- Call a function to set up a PWM that will be used to drive the LED (led_control_init)

- Initialize the two models

  - Light lightness server (register a callback)

  - The property server (no callback since property is read only)

mesh_app_message_handler:

- Only one event - WICED_BT_MESH_LIGHT_LIGHTNESS_SET

- Take lightness_actual_present and converts it to a percentage brightness as an 8-bit value

- Set the PWM

mesh_app_attention:

- Start/stop timer.

- Every 1 second, do whatever is necessary (invert LED, decrement remaining time)

Button Processing:

- No callback provided, so default behavior used – press button for factory reset.

## BLE_MESH_OnOffSwitch (Client)

- Basic on/off client

- Use with LightDimmable

### Configuration

| Models | WICED_BT_MESH_DEVICE WICED_BT_MESH_MODEL_ONOFF_CLIENT |
| --- | --- |
| Sensors | N/A |
| Properties | N/A |
| Features | WICED_BT_MESH_CORE_FEATURE_BIT_LOW_POWER |

### User Application Functionality

on_off_switch.c.

callbacks for application initialization (mesh_app_init) and button processing (mesh_app_hardware_init).

mesh_app_init:

- Just initializes the onoff client model

- The onoff model has no callback function - does not check the result of transmission messages or status messages

mesh_app_hardware_init:

Setup both edge interrupt on the button and register a callback:

button_interrupt_handler: This function does the following:

- Check if the button state has changed. If it has:

  - If the button is pressed, increment a counter

  - If the button is released, calculate the time that it was pressed:

    - If the button was pressed more than 15 seconds, do a factory reset.

    - Otherwise, call the process_button_push function.

process_button_push: This function sends a mesh message to toggle the on/off state

# Client Applications

- Need client application to provision/configure

## Client Control Mesh (Windows)

- Uses a kit with special FW to communicate to the mesh network.

- Works in versions of Windows before 10.

- The application has tabs for Configuration (provisioning, etc.), Models (for interacting with any model), and Light Control (specific for lighting applications).

## Mesh Client (Windows)

- Uses the PC's BLE radio to communicate to the mesh devices

- Needs Windows 10

- Create network, provision/configure devices, control lighting devices

## Mesh Lighting Controller (Android)

- Android app written by Cy. Not on Google Play Store (yet)

- The app can create mesh networks, provision/configure devices and can control lighting devices

## Sylvania Smart Home by LedVance (Android)

- Android app from Sylvania. On the Google Play Store

- iOS version of the app is for HomeKit – it will NOT work with BT Mesh

- Dimmer control does NOT work with our LightDimmable example.

# (Advanced) OTA

- The mesh library GATT definition includes the OTA upgrade service by default. Therefore, it is possible to update the firmware over BLE on an unprovisioned mesh device.

- Not able to get this to work yet.

# (Advanced) Apple HomeKit

Apple HomeKit uses a completely different protocol – BLE and WiFi, not mesh (yet)

Cypress FW will allow devices that support mesh to also support HomeKit in the following ways:

- (Short Term): Allow a single device to advertise as both an unprovisioned mesh device and an unpaired HomeKit device. The user can then choose to use it as a mesh device or a HomeKit device but not both at the same time.

- (Longer Term): Allow a single device to be controlled simultaneously by Mesh clients and HomeKit clients.