Chapter 7B: Bluetooth Mesh Protocol

# MESH Stack Architecture

## Bearer Layer

- Advertising or GATT (via proxy)

## Network Layer

- Defines message address types and message format

- Can have multiple interfaces including local to determine message routing between elements in the same node

- Input filter and output filters are used to determine if messages should be delivered up/down

- Relay and Proxy are implemented in the network layer

- It only decrypts using the NetKey, not the AppKey so it doesn't read the application data

## Lower Transport Layer

- Segmenting and reassembling

## Upper Transport Layer

- Encryption, decryption, and authentication of application data using the AppKey

- Deals with control messages (friendship and heartbeat messages)

## Access Layer

- Defines how higher-level applications can use the UTL. Format of app data, encryption and decryption used in the UTL, etc.

## Foundation Model Layer

- Device configuration and node health models

## Model Layer

- All other models besides the foundation models – generic, sensor, lighting, etc.

- This is the layer the user application interacts with